SPECIALIST REPORT

Vessel: Sailing Yacht Viento
Location of inspection & date: Puerto Vallarta, México,  18 June 2026
Surveyor/Technician: Ariel Mendoza fazio

Visual Inspection Findings

Wiring condition: A bundle of cables was found completely melted due to direct exposure to flames in the staircase area.

Affected systems: Power supply lines for the air conditioning system (Chiller Inverter) and the water heater. Both units are located beneath the saloon floorboards.

Technical Wiring Specifications

The affected lines have an approximate length of 6 meters with the following specifications:

- Chiller Inverter: OLFLEX CLASSIC 100 3G4 cable.

- Water Heater: OLFLEX CLASSIC 100 3G2.5 cable.

Routing and Connection Analysis

The power lines are not continuous from the main electrical panel to the equipment. They feature two connection points utilizing WAGO 770-113 male and female connectors:

- Point A (Origin of fire): Located underneath the saloon staircase. The connections in this specific area were found completely destroyed and charred.

- Point B: Located just before entering the equipment (outside the critical fire zone).

Evidence Retained for Analysis

The following components were removed from the vessel for detailed laboratory testing and analysis:

- Charred remains of the pluggable connections located under the stairs.

- Remaining sections of the OLFLEX cables (from the electrical panel to the equipment).

- Circuit breakers removed from the main electrical panel:

  - Hager MBN 116 B16 (16 Amps).

  - Hager MBN 120 B20 (20 Amps).

Conclusion

The physical evidence points to a poor electrical contact (loose connection) in one of the WAGO connectors located under the staircase. This fault caused high contact resistance, leading to progressive overheating (Joule effect) that ultimately triggered the fire in the supply lines, without the circuit breakers tripping in time to prevent it.

LEGAL PRINCIPLES

- This report is a true and exact copy of the report filed in the general archives of TALLAMAR ELECTRONICS, S.L.U. (from now on, TALLAMAR)

- TALLAMAR answers only in relation to the results presented in this report and referring exclusively to the materials, samples or equipment therein indicated. Unless otherwise expressly mentioned, the samples or equipment have been freely chosen and sent by the Client.

- TALLAMAR disclaims any responsibility regarding the interpretation of any undue use that might be made of this report.

- Total reproduction of this report, including the reproduction for advertising purposes, is prohibited without prior authorization of TALLAMAR in writing.

- Results are considered to be property of the requestor and TALLAMAR will not divulge them to any third parties without the requestor’s prior authorization.

- None of the indications herein included can be interpreted as a warranty for the commercial trademarks or the products/machinery analysed, whenever they are mentioned.

- The materials or samples used to perform the tests shall be kept at the Center for three months after the report is issued; after this time, it may be destroyed. Therefore, all verifications that the Client wished to carry out shall be done within said period of time.

This document is Signed and Stamped for and on behalf of

| Tallamar repres. | Xavier Raich |
| --- | --- |
| Charge | Owner / Expert technician |
| Signed and Stamped |  |